UNIT 10 — Introduction to Digital Electronics Physics · Form 4 · MSCE · Malawian Science Books

This unit introduces you to the electronics inside the devices you use every day — phones, radios, calculators, and computers. We start with what materials conduct electricity, then meet the components that control it, and finish with the logic gates that make computers think.

10.1  Conductors, Semiconductors and Insulators

THE BIG IDEA Materials are sorted by how easily electric charge moves through them. Conductors let charge flow freely. Insulators block it. Semiconductors are the middle — they can be made to conduct or not, and that control is what makes every electronic device possible.

Why this matters

Without semiconductors, there would be no phones, no radios, no computers. Malawi's own electronics repair shops depend on understanding these three classes.

The three classes of materials

| Class | Examples | Resistance | Behaviour |
| --- | --- | --- | --- |
| Conductors | Copper, aluminium, silver, graphite | Very low | Charge flows easily |
| Semiconductors | Silicon, germanium | Medium | Flows only under the right conditions |
| Insulators | Rubber, plastic, glass, dry wood, air | Very high | Charge does not flow |

Physical reason

In a conductor, some electrons are free to move — they are not tightly held by any one atom. In an insulator, every electron is tightly bound to its atom, so charge cannot move. In a semiconductor, electrons are held with medium strength — the right push (heat, light, or a voltage) can free them.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.1 — Energy band diagrams: insulator, semiconductor, conductor (showing the forbidden gap)

Resistance values (typical)

- Conductors: below 10⁻⁶ Ω·m (resistivity)

- Semiconductors: between 10⁻⁶ and 10⁴ Ω·m

- Insulators: above 10⁸ Ω·m

KEY POINTS — 10.1 - Conductors allow charge to flow; examples: copper, aluminium, graphite. - Insulators block charge; examples: rubber, plastic, glass. - Semiconductors sit between; examples: silicon, germanium. - The difference is how tightly electrons are held by their atoms. - Semiconductors are the foundation of every electronic device.

10.2  Intrinsic and Extrinsic Semiconductors

THE BIG IDEA A pure semiconductor is called intrinsic. On its own, it hardly conducts. But if we add a tiny amount of another element, it becomes extrinsic — and now it conducts in a controlled way. That single idea is what makes transistors and diodes possible.

Intrinsic semiconductors

Pure silicon or germanium. In a pure crystal at room temperature, a few electrons break free from their atoms and become free to move. Each freed electron leaves behind a hole — a place where an electron is missing, which acts like a positive charge carrier.

At room temperature, intrinsic silicon has very few free carriers. Its conductivity is low.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.2 — Pure silicon crystal lattice showing an electron–hole pair

Extrinsic semiconductors

We dope the pure semiconductor — add a very small amount (about 1 in 10⁶ atoms) of another element to increase the number of carriers. This is called doping.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.3 — Doped silicon showing extra free electron (n-type) or extra hole (p-type)

KEY POINTS — 10.2 - Intrinsic = pure semiconductor (silicon, germanium). - Extrinsic = doped semiconductor (has added impurities). - Free electrons and holes carry current in semiconductors. - A hole behaves like a positive charge. - Doping hugely increases conductivity.

10.3  Doping in Semiconductors

THE BIG IDEA Doping is adding a controlled impurity to silicon. Add an element with one extra electron → you get n-type (negative carriers). Add an element with one fewer electron → you get p-type (positive carriers).

n-type doping

Silicon has 4 valence electrons. If we add phosphorus or arsenic (which have 5), four of their electrons bond with neighbouring silicon atoms, and the fifth is free to move. This gives extra negative carriers → n-type semiconductor.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.4 — n-type doping: silicon + phosphorus (extra electron shown free)

p-type doping

If we add boron or aluminium (which have only 3 valence electrons), one bond is left incomplete — this is a hole. The hole behaves like a positive carrier. This is p-type semiconductor.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.5 — p-type doping: silicon + boron (hole shown)

KEY POINTS — 10.3 - n-type: doped with a 5-valence atom (P, As). Extra free electrons. - p-type: doped with a 3-valence atom (B, Al). Extra holes. - n-type is not negatively charged overall — it is still neutral. It just has more free electrons. - p-type is not positively charged overall. It just has more holes.

10.4  Electronic Components

THE BIG IDEA Electronic circuits are built from a small number of components. Each one has a job: resistors limit current, capacitors store charge, diodes let current flow one way only, transistors switch and amplify, LEDs give light, and ICs pack millions of parts into one chip.

The Resistor

A resistor opposes the flow of current. Its value is measured in ohms (Ω). We use it to protect components from too much current.

R = V / I

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.6 — Resistor symbol and a typical carbon-film resistor

The Capacitor

A capacitor stores electric charge. Its value is measured in farads (F). It charges up when connected to a supply and discharges when the supply is removed.

Q = C V

where Q = charge in coulombs (C), C = capacitance in farads (F), V = potential difference in volts (V).

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.7 — Capacitor symbol and a typical electrolytic capacitor

The Diode

A diode allows current to flow in one direction only. The arrow on its symbol shows the direction of conventional current. If the diode is connected the wrong way round, it blocks current.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.8 — Diode symbol, forward-biased and reverse-biased diagrams

The Light-Emitting Diode (LED)

An LED is a diode that gives out light when current passes through it. They are used in torches, phone screens, indicator lights, and solar lamps — common in Malawi.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.9 — LED symbol and an actual LED

The Transistor

A transistor has three terminals: base (B), collector (C), and emitter (E). A small current at the base controls a much larger current from collector to emitter. This makes it act as a switch or an amplifier.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.10 — npn transistor symbol with B, C, E labelled

The Integrated Circuit (IC)

An integrated circuit contains thousands (sometimes billions) of transistors, resistors, and diodes on a single tiny chip of silicon. The microprocessor in a phone is one example.

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.11 — IC chip and pin diagram (e.g. 555 timer or 7400 series)

KEY POINTS — 10.4 - Resistor — limits current; unit: ohm (Ω); R = V/I. - Capacitor — stores charge; unit: farad (F); Q = CV. - Diode — allows current one way only. - LED — a diode that emits light. - Transistor — three terminals (B, C, E); used as a switch or amplifier. - IC — many components on one silicon chip. - Circuit symbols are standard worldwide — learn them by heart.

10.5  Electronic Circuits and Signals

THE BIG IDEA Electronic signals come in two types: analogue (smoothly varying, like sound) and digital (only two states — HIGH or LOW, 1 or 0). Digital signals are how computers work.

Analogue vs digital signals

|  | Analogue | Digital |
| --- | --- | --- |
| Values | Continuous (any value) | Only two: 0 or 1 |
| Example | Sound, temperature | Switch, computer data |
| Noise | Easily distorted | Very resistant to noise |

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.12 — Analogue sine wave vs digital square wave (both on same axes for comparison)

Logic levels

In digital electronics we use two voltage levels:

- LOW = 0 V = logic 0

- HIGH = typically 5 V = logic 1

KEY POINTS — 10.5 - Analogue signals vary continuously. - Digital signals have only two levels: 0 and 1. - Digital signals are more resistant to noise and easier to store. - Logic 0 = LOW voltage; Logic 1 = HIGH voltage.

10.6  Logic Gates

THE BIG IDEA A logic gate is a tiny circuit that makes a decision based on its inputs. It takes 0s and 1s in, and gives one 0 or 1 out. Combining gates makes calculators, phones, and computers think. There are seven gates you must know.

The AND gate

Output is 1 only when BOTH inputs are 1.

Y = A · B

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.13 — AND gate symbol (D-shape) and its truth table

| A | B | Y = A·B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 1 |

The OR gate

Output is 1 when at least one input is 1.

Y = A + B

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.14 — OR gate symbol and its truth table

| A | B | Y = A+B |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 1 |

The NOT gate (inverter)

Output is the opposite of the input.

Y = A̅ (read: 'A bar' or 'NOT A')

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.15 — NOT gate symbol (triangle with circle) and truth table

| A | Y = NOT A |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

The NAND gate

The opposite of AND. Output is 1 unless both inputs are 1.

Y = (A · B) with bar — read: NOT (A AND B)

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.16 — NAND gate symbol and truth table

| A | B | Y = NAND |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

The NOR gate

The opposite of OR. Output is 1 only when both inputs are 0.

Y = NOT (A + B)

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.17 — NOR gate symbol and truth table

| A | B | Y = NOR |
| --- | --- | --- |
| 0 | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 0 | 0 |
| 1 | 1 | 0 |

The XOR gate (Exclusive-OR)

Output is 1 when the inputs are different.

Y = A ⊕ B

[ IMAGE PLACEHOLDER ] → Click here and insert your image →

Figure 10.18 — XOR gate symbol and truth table

| A | B | Y = XOR |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 0 |

KEY POINTS — 10.6 - AND: Y = 1 only when both inputs are 1. - OR: Y = 1 when at least one input is 1. - NOT: Y = opposite of A. - NAND: opposite of AND. - NOR: opposite of OR. - XOR: Y = 1 when inputs are different. - Learn each symbol and its truth table by heart — they are guaranteed MSCE marks.

Worked Examples

Example 1 — Using Ohm's law with a resistor

A resistor is connected to a 6 V supply. A current of 0.5 A flows. Find its resistance.

Step 1. Write down what you know: V = 6 V, I = 0.5 A.

Step 2. Use R = V / I.

R = 6 / 0.5

⌈  R = 12 Ω  ⌉

Example 2 — Charge stored in a capacitor

A 100 μF capacitor is charged to 10 V. Calculate the charge stored.

Step 1. Convert μF to F: 100 μF = 100 × 10⁻⁶ F.

Step 2. Use Q = CV.

Q = (100 × 10⁻⁶) × 10

⌈  Q = 1 × 10⁻³ C = 1 mC  ⌉

Example 3 — Truth table from a logic circuit

A NAND gate has inputs A = 1 and B = 0. What is the output?

NAND is the opposite of AND. AND of 1 and 0 is 0. Therefore NAND is 1.

⌈  Y = 1  ⌉

Check Yourself

1. Give two examples each of conductors, semiconductors, and insulators. 2. What is the difference between an intrinsic and an extrinsic semiconductor? 3. What is a hole in a semiconductor? 4. Name the element used to dope silicon to make n-type. What does it add? 5. Name the element used to dope silicon to make p-type. What does it add? 6. State the function of a diode. 7. How many terminals does a transistor have? Name them. 8. What is the difference between an analogue and a digital signal? 9. Write the truth table for an AND gate. 10. Write the truth table for a NOR gate.

Exam-Style Questions (MSCE)

Q1. (a) Distinguish between a conductor, a semiconductor, and an insulator, giving one example of each. (b) Explain why silicon is more useful than copper in making computer chips. Q2. (a) What is meant by doping? (b) Describe how silicon is doped to make an n-type semiconductor. (c) What are the majority charge carriers in a p-type semiconductor? Q3. Draw the circuit symbol for each of the following and state its function: (a) resistor (b) capacitor (c) diode (d) LED (e) transistor. Q4. Define the term truth table. Hence construct the truth table for a two-input XOR gate. Q5. A resistor is connected across a 12 V battery. A current of 0.4 A flows. (a) Calculate the resistance of the resistor. (b) Calculate the power dissipated in the resistor. Q6. A 470 μF capacitor is charged to 25 V. (a) Calculate the charge stored. (b) State what happens when the capacitor is disconnected from the supply and connected to a small lamp. Q7. A logic circuit is made of an AND gate whose output is fed into a NOT gate. Both inputs of the AND gate are set to 1. (a) What is the output of the AND gate? (b) What is the final output of the circuit? (c) Name this combined gate. Q8. Explain the difference between a digital and an analogue signal, giving one example of each. Q9. A NOR gate has inputs A = 1 and B = 1. What is the output? Justify your answer. Q10. (a) What is an integrated circuit? (b) Give two advantages of using integrated circuits instead of separate components.

Full Worked Solutions

Check Yourself — Answers

1. Conductors: copper, aluminium. Semiconductors: silicon, germanium. Insulators: rubber, glass. 2. Intrinsic = pure; extrinsic = doped with an impurity. 3. A hole is a missing electron in the crystal lattice; it behaves like a positive charge. 4. Phosphorus (or arsenic); it adds a free electron. 5. Boron (or aluminium); it creates a hole. 6. A diode allows current to flow in one direction only. 7. Three: base (B), collector (C), emitter (E). 8. Analogue varies continuously; digital has only two levels (0 and 1). 9. AND: 0-0→0, 0-1→0, 1-0→0, 1-1→1. 10. NOR: 0-0→1, 0-1→0, 1-0→0, 1-1→0.

Exam-Style — Full Solutions

Q1. (a) Conductor: charge flows easily (copper). Semiconductor: flow only under certain conditions (silicon). Insulator: charge does not flow (rubber). (b) Silicon's conductivity can be controlled by doping, so it can act as a switch — that is essential for chips. Copper cannot be switched. Q2. (a) Doping is the addition of a small, controlled amount of impurity to a pure semiconductor. (b) Silicon is doped with a 5-valence element such as phosphorus. Four electrons bond and the fifth becomes free — this gives extra negative carriers. (c) Holes. Q3. Symbols must be drawn. Functions: resistor — opposes current; capacitor — stores charge; diode — allows current one way; LED — emits light; transistor — switches/amplifies. Q4. A truth table lists all possible input combinations and the corresponding output. XOR: 0-0→0, 0-1→1, 1-0→1, 1-1→0. Q5. (a) R = V/I = 12/0.4 = 30 Ω. (b) P = VI = 12 × 0.4 = 4.8 W. Q6. (a) Q = CV = 470 × 10⁻⁶ × 25 = 1.175 × 10⁻² C ≈ 11.75 mC. (b) The capacitor discharges through the lamp, lighting it briefly. Q7. (a) AND output = 1. (b) NOT of 1 = 0. (c) NAND. Q8. Digital: only two levels (0 or 1) — e.g. computer data. Analogue: varies continuously — e.g. sound or temperature. Q9. NOR of 1 and 1 is 0. OR of 1 and 1 is 1; NOT gives 0. Q10. (a) An IC is a complete circuit with many components built on one silicon chip. (b) Advantages: smaller size, lower cost, higher reliability, faster.

Common Mistakes

- Confusing p-type with positively charged. It is neutral; it just has more holes. - Saying n-type is negatively charged. It is neutral; it just has more free electrons. - Mixing up the diode and LED symbols — the LED has two arrows pointing out. - Forgetting that the transistor's base current is small; it controls a larger collector current. - Writing AND as 'A plus B' — AND is multiply (·), OR is add (+). - Drawing NAND and NOR symbols without the small circle at the output — that circle means NOT. - Confusing analogue and digital: analogue is a smooth wave, digital is a square wave.

EXAM TIP MSCE examiners reliably ask for symbols and truth tables. Draw every symbol cleanly, with all arrows and labels. For truth tables, always write the header row (A, B, Y). Missing the header loses easy marks. When asked to explain differences (conductor vs semiconductor), write a cause-and-effect sentence: 'Silicon conducts only when doped because its electrons are held with medium strength.'

REMEMBER • Semiconductors (doped) are the heart of all electronics. Learn the 7 logic gates and their truth tables by heart — free marks in MSCE.

Unit Test 10

SECTION A — Multiple Choice (5 marks) 1. Which of the following is a semiconductor? A. Copper B. Rubber C. Silicon D. Aluminium 2. A hole in a semiconductor behaves like: A. A negative charge B. A positive charge C. A neutral particle D. A photon 3. The output of an AND gate is 1 when: A. Any input is 1 B. All inputs are 1 C. All inputs are 0 D. Inputs are different 4. The base current of a transistor is 0.1 mA and the collector current is 10 mA. The current gain is: A. 0.1 B. 10 C. 100 D. 1000 5. Which component stores electric charge? A. Resistor B. Diode C. Capacitor D. Transistor

SECTION B — Short Answers (10 marks) 6. State two differences between a conductor and an insulator. (2) 7. Explain what is meant by doping and give one example of an n-type dopant. (2) 8. Draw the circuit symbol for a diode and explain its function. (2) 9. Write the truth table for a NAND gate. (2) 10. Give two advantages of integrated circuits over separate components. (2)

SECTION C — Structured Questions (15 marks) 11. A student sets up a circuit containing a 9 V battery, a resistor, and an ammeter. The ammeter reads 0.3 A. (a) Calculate the resistance of the resistor. (3) (b) Calculate the power dissipated in the resistor. (2) 12. (a) Draw the truth table for a two-input OR gate. (3) (b) A logic circuit consists of an OR gate feeding into a NOT gate. Name this combination gate and write its truth table. (4) 13. A 220 μF capacitor is charged from a 12 V supply. (a) Calculate the charge stored on the capacitor. (3) (b) Describe what happens when the capacitor is then connected across a small lamp. (2)

END OF UNIT 10 You have now covered the full Introduction to Digital Electronics unit. Next unit: Electromagnetic Spectrum (Unit 11).