Electricity & Circuits Physics · Form 4 · MSCE · Patch PHY_F4_01

1. The Big Idea

Electricity is just electrons moving through a wire. When they move, they carry energy. If we control that movement with a circuit, we can light a bulb, run a phone, or cook nsima. Every circuit is a loop — the current must have a path out and a path back. ---

2. Simple Explanation

Think of a bicycle chain. The pedals push the chain, the chain turns the wheel. If the chain breaks, nothing moves.

An electric circuit works the same way:
- The battery = your legs (pushes the electrons)
- The wire = the chain (carries them)
- The bulb/motor = the wheel (does the useful work)
- The switch = a brake that stops the chain

If any part breaks, the whole loop stops. That is why a broken bulb in a series circuit switches off the whole string.

### The three key quantities

| Quantity | Symbol | Unit | Meaning |
| --- | --- | --- | --- |
| Current | I | Ampere (A) | How much charge flows per second |
| Voltage | V | Volt (V) | The "push" given to the electrons |
| Resistance | R | Ohm (Ω) | How much the component slows the flow |

### Ohm's Law

$$V = I \times R$$

Rearranged:
- $I = V / R$
- $R = V / I$

### Series vs Parallel

Series: one path. Current is the same everywhere. Voltages add.
$$R_{total} = R_1 + R_2 + R_3$$

Parallel: many paths. Voltage is the same across each branch. Currents add.
$$\frac{1}{R_{total}} = \frac{1}{R_1} + \frac{1}{R_2} + \frac{1}{R_3}$$

### Electrical Power

$$P = V \times I$$

Also useful:
- $P = I^2 R$
- $P = V^2 / R$

Unit of power: Watt (W). Energy used: $E = P \times t$ (in Joules, or kWh for bills).

---

3. Key Points Current is the same everywhere in a series circuit. Voltage is the same across every branch in a parallel circuit. Ohm's Law: $V = IR$. Learn all three rearrangements. Series resistors add up. Parallel resistors always give a smaller total than the smallest one. Power: $P = VI$. Energy: $E = Pt$. Ammeters go in series with a component. Voltmeters go in parallel across a component.

4. Worked Example Question: A 12 V battery is connected to two resistors in series: $R_1 = 4\,\Omega$ and $R_2 = 8\,\Omega$. Find (a) total resistance, (b) current, (c) voltage across each resistor, (d) power used by $R_2$. Solution: (a) Total resistance (series): $$R_{total} = R_1 + R_2 = 4 + 8 = 12\,\Omega$$ (b) Current (Ohm's Law): $$I = \frac{V}{R} = \frac{12}{12} = 1\,\text{A}$$ (c) Voltage across each resistor: - $V_1 = I \times R_1 = 1 \times 4 = 4\,\text{V}$ - $V_2 = I \times R_2 = 1 \times 8 = 8\,\text{V}$ - Check: $4 + 8 = 12\,\text{V}$ ✅ (matches the battery) (d) Power in $R_2$: $$P = V_2 \times I = 8 \times 1 = 8\,\text{W}$$ ---

5. Check Yourself

1. What is the unit of resistance?

2. In a series circuit, is the current the same or different at each point?

3. A 6 V battery drives 2 A through a resistor. What is its resistance?

4. Two 10 Ω resistors are in parallel. What is the total resistance?

5. Write Ohm's Law in three forms.

*(Answers at the end.)*

---

6. Exam-Style Questions (MSCE)

Q1. A student connects three resistors of $2\,\Omega$, $3\,\Omega$, and $5\,\Omega$ in series to a 20 V supply. Calculate:
(a) the total resistance,
(b) the current in the circuit,
(c) the voltage across the $5\,\Omega$ resistor,
(d) the power dissipated in the $3\,\Omega$ resistor.

Q2. Two resistors, $R_1 = 6\,\Omega$ and $R_2 = 3\,\Omega$, are connected in parallel across a 12 V battery.
(a) Find the total resistance.
(b) Find the total current from the battery.
(c) Find the current through each resistor.
(d) Show that the currents add to your answer in (b).

Q3. Explain why the total resistance of resistors in parallel is always less than the smallest individual resistor.

Q4. An electric iron is rated 240 V, 1000 W.
(a) Calculate the current it draws.
(b) Calculate its resistance.
(c) If used for 30 minutes, how much energy (in kWh) does it use?

Q5. A 12 V car battery is connected to a bulb of resistance $6\,\Omega$ and an ammeter. Draw the circuit, then calculate the current. Where must the ammeter be placed, and why?

---

7. Answers

Check Yourself:
1. Ohm (Ω)
2. The same
3. $R = V/I = 6/2 = 3\,\Omega$
4. $1/R = 1/10 + 1/10 = 2/10$ → $R = 5\,\Omega$
5. $V = IR$, $I = V/R$, $R = V/I$

Q1.
(a) $R = 2 + 3 + 5 = 10\,\Omega$
(b) $I = 20/10 = 2\,\text{A}$
(c) $V = 2 \times 5 = 10\,\text{V}$
(d) $P = I^2 R = 2^2 \times 3 = 12\,\text{W}$

Q2.
(a) $1/R = 1/6 + 1/3 = 1/6 + 2/6 = 3/6$ → $R = 2\,\Omega$
(b) $I = 12/2 = 6\,\text{A}$
(c) $I_1 = 12/6 = 2\,\text{A}$; $I_2 = 12/3 = 4\,\text{A}$
(d) $2 + 4 = 6\,\text{A}$ ✅

Q3. In parallel, each resistor gives the current *another* path. More paths = easier flow = less overall resistance. The total can never be smaller than the smallest path alone; adding more paths only makes it easier still.

Q4.
(a) $I = P/V = 1000/240 \approx 4.17\,\text{A}$
(b) $R = V/I = 240/4.17 \approx 57.6\,\Omega$ (or $R = V^2/P = 57600/1000 = 57.6\,\Omega$)
(c) $E = P \times t = 1\,\text{kW} \times 0.5\,\text{h} = 0.5\,\text{kWh}$

Q5. Circuit: battery → ammeter → bulb → back to battery (series loop). $I = V/R = 12/6 = 2\,\text{A}$. The ammeter must be in series because it measures the current *through* a component — all the current must pass through it.

---

8. Exam Tip Examiners love asking students to compare series and parallel in the same question. Always write down what stays the same: > - Series → current is the same, voltages add > - Parallel → voltage is the same, currents add > > Also: if your final answer has the wrong unit, you lose a mark even if the number is right. Always write the unit. ---

REMEMBER • A circuit is a loop. No loop, no current. Ohm's Law is your friend: V = IR — and the two rearrangements must be automatic.